How to use Http

The HTTP protocol support TCP Client Mode and support communication interface with RJ45 and WIFI and 4G.

Step 1: Open Net interface.

Open ReaderSoft file path

Double click ReaderSoft.exe logo.

, Choose connect port, such as COM, USB or Net(RJ45)

Then click Open button, it will show like this picture

Pic 1

Select Interface is WIFI, or RJ45 or 4G, and select WorkMode is ActiveMode.

After set over, click Set button, if show paramters error, pls click read button first then set .

Step 2: Set Remote IP and port

DNS mean open server domain mode, if open DNS, please input domain name such as www.google.com not with http:// , and RemoteIP is invalid.  And it not support HTTPS encrypt web.

After set over, click Set button, if show paramters error, pls click read button first then set .

For reduce data package and speed up transmission, you can set filterTime, set up zero mean open filterTime.

ParamSet -> FilterTime

Step 3: Open HTTP protocol

In the AdvanceSet

Select Protocol and select 4,HTTP

And input HttpParam, such as /read?

Click Set button and HTTP begin work.

Http procotol pls open HTTP protocol file to read.